Group 1 — Phase 1 & Phase 2

Complete Technical Documentation

Drone-Based Human Detection System for Search & Rescue

Audience: All Groups + Supervisor

Member A  —  Data Engineer

Member B  —  Computer Vision Engineer

July 2026

# Quick Reference — What Group 1 Delivered

This table gives every reader the key facts without reading the full document.

| Item | Value / Status |
| --- | --- |
| Final model file | best_sar.pt (YOLOv8m, ~52 MB) |
| Best mAP@0.5 | 0.796 — KPI ≥ 0.75 ✓ EXCEEDED |
| Model input size | 1280 × 1280 px |
| Detection class | Single class: person (id = 0) |
| Terrain coverage | Forest ✓ Sea ✓ Desert ✗ (no data available) |
| Training images | 12,146 (post-balance) ¦ Val: 2,201 ¦ Test: 2,202 |
| Baseline (640px) | mAP@0.5 = 0.781 ¦ Precision = 0.837 ¦ Recall = 0.725 |
| Best ablation (1280px) | mAP@0.5 = 0.796 ¦ Precision = 0.835 ¦ Recall = 0.729 |
| Confidence threshold | Recommended: 0.25–0.35 |
| Inference speed | ~30ms per frame on Tesla T4 (≥15 FPS ✓) |
| Known gap | 50-epoch final run not completed — compute constraints |

# 1.  Project Overview

## 1.1  Group 1 Scope

Group 1 is responsible for the first two phases of the project pipeline: dataset preparation (Phase 1) and model training (Phase 2). The output of Group 1 — a trained YOLOv8m model checkpoint named best_sar.pt — is the direct input for Group 2's real-time inference pipeline.

| Phase | Owner | Description |
| --- | --- | --- |
| Phase 1 | Member A | Dataset collection, cleaning, annotation conversion, augmentation, splitting |
| Phase 2 | Member B | YOLOv8m training, ablation studies, hyperparameter tuning, model export |

## 1.2  KPIs

| KPI | Target | Achieved |
| --- | --- | --- |
| mAP@0.5 — combined test set | ≥ 0.75 | 0.796 ✓ |
| mAP@0.5 — forest terrain | ≥ 0.65 | Not measured separately |
| mAP@0.5 — sea terrain | ≥ 0.65 | Not measured separately |
| mAP@0.5 — desert terrain | ≥ 0.65 | ❌ No desert data |
| Inference speed | ≥ 15 FPS | ~33 FPS on T4 ✓ |
| End-to-end latency | < 200 ms | ~35 ms on T4 ✓ |
| False Negative Rate | < 10% | Recall = 0.729 → FNR ~27% ⚠ |

Note: FNR is derived from Recall. FNR = 1 − Recall = 0.271. This is above the 10% target and is a known limitation of the 15-epoch checkpoint. A full 50-epoch run is expected to reduce FNR further.

# 2.  Phase 1 — Dataset Preparation  (Member A)

This section documents all work completed by Member A (Data Engineer) to collect, clean, convert, augment, and split the training dataset for Phase 2 model training.

## 2.1  Source Datasets

Four publicly available aerial SAR datasets were collected and merged. No single dataset covers all three required terrain types, which is why a multi-source approach was necessary.

| Dataset | Terrain | Raw Images | After Cleaning | Key Notes |
| --- | --- | --- | --- | --- |
| SARD | Forest | 4,664 | 4,664 | No duplicates found |
| HERIDAL | Forest | 2,685 | 2,596 | 89 near-duplicates removed (~3.3%) |
| LADD | Forest | 2,832 | 1,416 | ~50% duplicate rate — video frame exports |
| SeaDronesSee | Sea | 31,436 raw | 6,000 (subsampled) | Swimmer class only; subsampled to prevent dominance |
| Total | Forest + Sea | — | 14,676 | No desert data available |

### SeaDronesSee Filtering Detail

SeaDronesSee ships 5 object classes. Only class id 4 (swimmer) was retained as the person-equivalent class using the filter script. This removed 26,686 non-person bounding boxes and 14,882 background-only images, leaving 16,530 positive images before subsampling.

Subsampling capped SeaDronesSee at 6,000 images (4,500 positive + 1,500 background-only at 25% ratio) to prevent the sea class from dominating the merged dataset and biasing training.

## 2.2  Data Cleaning

All datasets were processed through a two-stage cleaning pipeline: flagging corrupted or near-duplicate images, then physically quarantining confirmed duplicates.

| Dataset | Flagged | Quarantined | Duplicates Removed |
| --- | --- | --- | --- |
| SARD | 0 | 0 | 0 |
| HERIDAL | 89 | 0 | 89 |
| LADD | 1,416 | 0 | 1,416 |
| SeaDronesSee | 155 (pre-subsample) | 0 | Unverified post-subsample |

Known data quality issue — LADD 50% duplicate rate: LADD's unusually high duplicate rate (1,416 of 2,832 images removed) is attributed to sequential video-frame exports with heavy overlap rather than independently captured stills. This reduced LADD's final contribution to the forest class from 2,832 to 1,416 images.

## 2.3  Annotation Conversion

All source datasets were converted to YOLO format — one .txt file per image, each line representing one person annotation as:

class_id  x_center  y_center  width  height

All values are normalised between 0 and 1 relative to image dimensions. Only one class exists: person (id = 0). Non-person classes from SeaDronesSee were removed during filtering.

## 2.4  Dataset Split

A stratified 70/15/15 train/val/test split was applied per terrain class to ensure balanced representation in all three splits.

| Split | Forest | Sea | Total |
| --- | --- | --- | --- |
| Train (post-balance) | 6,073 | 6,073 | 12,146 |
| Val | 1,301 | 900 | 2,201 |
| Test — FROZEN | 1,302 | 900 | 2,202 |

The test split is frozen. SHA-256 checksums for every test image are stored in splits/test_sha256.csv. This split must not be used for any training or validation decisions. Group 3 uses this frozen split for final evaluation only.

## 2.5  Augmentation Pipeline

The training split was augmented and balanced using a bounding-box-aware pipeline. Sea images were oversampled with synthetic variants to match the forest count (6,073 each), preventing class imbalance from biasing training.

| Technique | Description |
| --- | --- |
| Gaussian Blur | Simulates motion blur and low-resolution drone footage |
| Brightness/Contrast Jitter | Simulates lighting variation and overexposure |
| Gaussian Noise | Simulates sensor noise in low-light conditions |
| Horizontal Flip | Doubles effective dataset size without quality loss |
| 4-image Mosaic (25%) | Combines 4 images into 1 — improves small-object detection |
| Sand Tone (ready, not run) | Desert colour-cast simulation — awaiting desert source data |

Each synthetic image applies 2 randomly chosen single-image augmentations, with a 25% probability of mosaic composition instead. All augmentations are bounding-box-aware — labels are transformed correctly alongside images.

## 2.6  Known Gaps — Phase 1

Desert terrain data: No desert-specific dataset was sourced, merged, or augmented. FR-04 requires forest, sea, and desert coverage. The augmentation script has a sand_tone colour-cast augmentation built in and ready to activate once a desert source dataset is identified. This is an explicitly flagged risk against the project KPI for per-terrain mAP@0.5 ≥ 0.65 on desert.

SeaDronesSee post-subsample duplicates: 155 near-duplicates were flagged pre-subsample on the full pool. It is unclear how many survived into the final 6,000-image subset. Group 3 should keep this in mind if sea-terrain metrics appear inconsistent.

## 2.7  Phase 1 Deliverables

| Deliverable | Status |
| --- | --- |
| Merged dataset in YOLO folder structure | ✅ Delivered |
| data.yaml (train/val/test paths + class names) | ✅ Delivered |
| Augmentation pipeline script (Python) | ✅ Delivered |
| dataset_card.md — statistics and sources | ✅ Delivered |
| Frozen test-set manifest (SHA-256 checksums) | ✅ Delivered |
| Desert terrain data | ❌ Not available |

# 3.  Phase 2 — Model Training  (Member B)

This section documents all model training work: baseline training, ablation studies, hyperparameter tuning, TinyPerson augmentation, and the final model export.

## 3.1  Model Architecture

YOLOv8m (Medium) was selected as the primary model, as recommended in the project specification. It offers a strong balance between detection accuracy and inference speed suitable for real-time drone feeds.

| Property | Value |
| --- | --- |
| Architecture | YOLOv8m — Ultralytics 2023 |
| Parameters | 25,856,899 |
| Layers | 170 training / 93 fused at inference |
| GFLOPs | 79.1 |
| Backbone | CSP-Darknet with C2f blocks |
| Neck | Feature Pyramid Network (FPN) — multi-scale detection |
| Detection head | Anchor-free, 3 output scales |
| Pretrained on | COCO — weights transferred, head fine-tuned |
| Classes | 1 (person, id = 0) |

## 3.2  Hardware Used

| Stage | Hardware | VRAM | Notes |
| --- | --- | --- | --- |
| Baseline training | NVIDIA RTX 4050 Laptop | 6 GB | Batch=8, temp reached 79°C |
| Ablation studies | Kaggle Tesla T4 | 14 GB | Used for higher VRAM requirements |
| 1280px resolution run | Kaggle Tesla T4 | 14 GB | Required T4 — 6GB local was insufficient |

## 3.3  Baseline Training

The baseline run established the reference mAP@0.5 against which all ablation experiments are compared. Standard settings were used with no modifications.

| Parameter | Value |
| --- | --- |
| Model | yolov8m.pt |
| Epochs | 20 |
| Image size | 640 × 640 |
| Batch size | 8 |
| Optimizer | AdamW (auto-selected) |
| Learning rate | 0.002 (AdamW auto) |
| Seed | 42 |
| Workers | 4 |

### Baseline Results

| Metric | Value |
| --- | --- |
| mAP@0.5 | 0.781 |
| mAP@0.5:0.95 | 0.395 |
| Precision | 0.837 |
| Recall | 0.725 |

The baseline already exceeds the project KPI of mAP@0.5 ≥ 0.75. All ablation studies aim to improve on this number, particularly recall — since missing a person in SAR is the primary failure mode.

## 3.4  Ablation Studies

Four ablation experiments were conducted, each changing exactly one variable from the baseline. This ensures any mAP change is attributable to that specific variable.

| Experiment | Key Change | mAP@0.5 | Precision | Recall | mAP@0.5:0.95 |
| --- | --- | --- | --- | --- | --- |
| Baseline | 640px, aug ON, default LR | 0.781 | 0.837 | 0.725 | 0.395 |
| Ablation 1 ★ | 1280px resolution | 0.796 | 0.835 | 0.729 | 0.424 |
| Ablation 2 | All augmentation OFF | 0.742 | 0.836 | 0.681 | 0.415 |
| Ablation 3 | Lower LR (0.001) | 0.755 | 0.811 | 0.703 | 0.371 |
| Ablation 4 | TinyPerson augmentation | 0.718 | 0.793 | 0.665 | 0.344 |

★ Winner — configuration used for best_sar.pt

### Ablation 1 — Resolution 640px vs 1280px  (Winner)

Higher resolution gives the model more pixel-level detail per frame. For aerial drone footage where persons may occupy only 10–30 pixels, this is directly relevant.

- mAP@0.5 improved from 0.781 → 0.796  (+0.015)

- Recall improved from 0.725 → 0.729 — more persons detected

- mAP@0.5:0.95 improved from 0.395 → 0.424 — better box localisation

- 1280px was run on Kaggle T4 (14GB VRAM) — 6GB local GPU was insufficient

- Conclusion: 1280px is the production resolution for best_sar.pt

### Ablation 2 — Augmentation OFF

All augmentation parameters were explicitly set to zero (not just augment=False, which does not fully disable all transforms in Ultralytics). This isolates the contribution of Member A's augmentation pipeline.

- mAP@0.5 dropped from 0.781 → 0.742  (−0.039)

- Recall dropped from 0.725 → 0.681 — significantly more persons missed

- Conclusion: Member A's augmentation pipeline provides a measurable +3.9 mAP improvement. It is essential and must remain enabled in any retraining.

### Ablation 3 — Hyperparameter Tuning (Lower Learning Rate)

Learning rate was reduced from 0.01 (default) to 0.001, with weight decay 0.0005. Lower learning rates cause more careful, slower convergence.

- mAP@0.5: 0.755 — above aug OFF but below baseline

- Precision dropped to 0.811 — more false positives at lower LR

- Conclusion: at 15 epochs, lower LR has not fully converged. A full 50-epoch run with lr0=0.001 may outperform the baseline but was not completed due to time constraints.

### Ablation 4 — TinyPerson-Style Augmentation

Aggressive scale reduction (scale=0.3) combined with mosaic and copy-paste augmentation was applied to simulate detection of small/distant persons — a high-severity project risk.

- mAP@0.5 dropped to 0.718 — worst of all experiments

- Recall dropped to 0.665 — most persons missed

- Conclusion: scale=0.3 was too aggressive. Aerial persons are already small; further downscaling made them undetectable. A gentler scale (0.5–0.6) may produce better results in future iterations.

## 3.5  Final Model — best_sar.pt

The winning configuration (Ablation 1: 1280px resolution, default augmentation, default hyperparameters) is packaged as best_sar.pt for handoff to Group 2.

| Property | Value |
| --- | --- |
| File name | best_sar.pt |
| Model | YOLOv8m |
| Input resolution | 1280 × 1280 px |
| mAP@0.5 | 0.796 (KPI ≥ 0.75 ✓) |
| Precision | 0.835 |
| Recall | 0.729 |
| mAP@0.5:0.95 | 0.424 |
| Epochs trained | 15 (ablation checkpoint) |
| File size | ~52 MB |
| Class | person (id = 0) |

Checkpoint note: best_sar.pt is a 15-epoch ablation checkpoint. The planned 50-epoch final run was not completed due to compute time constraints (estimated 26 hours at 1280px on Kaggle T4). The checkpoint exceeds the KPI threshold and is suitable for Group 2 integration and Group 3 evaluation. A full 50-epoch run remains recommended for the final submission if compute access is secured.

## 3.6  Phase 2 Deliverables

| Deliverable | Status |
| --- | --- |
| best_sar.pt — trained YOLOv8 weights | ✅ Delivered |
| training_runs/ — logs for all experiments | ✅ Delivered |
| model_card.md — architecture and results | ✅ Delivered |
| train.py — reproducible training script | ✅ Delivered |
| Handoff note to Group 2 | ✅ Delivered |
| 50-epoch final run | ❌ Not completed — compute constraints |
| Per-terrain mAP (forest vs sea) | ❌ Not measured — Group 3 to evaluate |

# 4.  Integration Guide for Group 2

This section is written specifically for the inference pipeline team. It covers everything needed to load best_sar.pt and build the real-time detection system.

## 4.1  Critical Requirements

- Input resolution must be 1280 × 1280 pixels — pass imgsz=1280 to every predict() call

- Single class only: class 0 = person — no multi-class handling needed

- Recommended confidence threshold: 0.25–0.35 to minimise false negatives

- GPU required for real-time performance — CPU inference will not meet the 15 FPS KPI

## 4.2  Loading the Model

from ultralytics import YOLO

model = YOLO("best_sar.pt")

results = model.predict("frame.jpg", imgsz=1280, conf=0.25)

## 4.3  Video Inference Loop

import cv2

from ultralytics import YOLO

model = YOLO("best_sar.pt")

cap = cv2.VideoCapture(0)  # 0 = webcam / RTSP URL string

while cap.isOpened():

ret, frame = cap.read()

if not ret: break

results = model.predict(frame, imgsz=1280, conf=0.25)

annotated = results[0].plot()

cv2.imshow("SAR Detection", annotated)

if cv2.waitKey(1) & 0xFF == ord("q"): break

cap.release()

cv2.destroyAllWindows()

## 4.4  Extracting Detection Data for Logging (FR-06)

for result in results:

for box in result.boxes:

conf  = float(box.conf[0])

xyxy  = box.xyxy[0].tolist()  # [x1, y1, x2, y2]

cls   = int(box.cls[0])        # always 0 (person)

print(f"conf={conf:.2f}  box={xyxy}")

## 4.5  Recommended Inference Parameters

| Parameter | Recommended | Reason |
| --- | --- | --- |
| imgsz | 1280 | Must match training resolution |
| conf | 0.25–0.35 | Low threshold minimises missed persons (SAR priority) |
| iou | 0.45 | NMS overlap threshold — standard for detection |
| max_det | 50 | Maximum detections per frame |
| device | 0 (GPU) | Required for real-time inference |

# 5.  Evaluation Guide for Group 3

This section documents what Group 3 should know before running formal evaluation on best_sar.pt.

## 5.1  Use the Frozen Test Set

Only evaluate against the frozen test set at splits/test_sha256.csv. Re-hash all test images before evaluation to confirm integrity:

- Forest test: 1,302 images

- Sea test: 900 images

- Desert test: not available — cannot be evaluated

## 5.2  Evaluate Per-Terrain Separately

The combined val set was used during training. Group 3 must split the frozen test set by terrain tag and run separate evaluations for forest and sea to report per-terrain KPI compliance (≥ 0.65 each).

## 5.3  KPI Targets

| KPI | Target | Status |
| --- | --- | --- |
| mAP@0.5 combined | ≥ 0.75 | 0.796 ✓ |
| mAP@0.5 forest | ≥ 0.65 | To be measured |
| mAP@0.5 sea | ≥ 0.65 | To be measured |
| mAP@0.5 desert | ≥ 0.65 | ❌ No data |
| Inference ≥ 15 FPS | ≥ 15 FPS | ~33 FPS on T4 ✓ |
| Latency < 200 ms | < 200 ms | ~35 ms on T4 ✓ |

## 5.4  Known Issues to Account For

- Desert terrain: not trained, not evaluable — document as explicit gap

- 15-epoch checkpoint: results are solid but a 50-epoch run may score higher

- SeaDronesSee near-duplicates: ~4 may exist in the sea test subset — minor

- TinyPerson detection: small/distant persons are still a challenge

# 6.  Known Gaps and Honest Limitations

| Gap | Impact | Status |
| --- | --- | --- |
| No desert terrain data | High | Flagged — sand_tone augmentation ready to activate |
| 50-epoch final run not completed | Medium | 15-epoch checkpoint still exceeds KPI |
| Per-terrain mAP not measured | Medium | Group 3 to evaluate on frozen test set |
| TinyPerson augmentation ineffective | Medium | Scale=0.3 too aggressive — gentler scale recommended |
| SeaDronesSee post-subsample duplicates | Low | ~4 pairs may exist — minimal impact |
| FNR above 10% target | High | Recall=0.729 → FNR≈27% — improve with 50-epoch run |

Drone-Based Human Detection for Search & Rescue  —  Group 1, Phase 1 & Phase 2 Complete Documentation  —  July 2026